# DANGEROUS CARGOES

Dangerous cargoes are classed as cargoes which owing to' their inflammable or explosive nature are liable to spontaneous combustion, either in themselves or when stowed with other cargoes.

Cargoes which give off explosive gases, poisonous fumes2 or tainting odours are also classed as dangerous goods, the gases of which, when mixed with air, are liable to explosion.

The following liquid cargoes may be classed as dangerous ones: fuel oil3, petrol, kerosene, lubricants, acids.

Such oil products as petrol, kerosene and fuel oil are transported in oil tankers, cisterns, iron casks, containers and cans.

Lubricating oils are transported in wooden barrels, in drums made of plywood, in cans and boxes.

Bitumen are usually made in blocks and as a rule in wooden or iron packages.

Cargoes which may readily ignite4 on contact with air should be packed in hermetically sealed receptacles, some of which may be required to be filled with suitable liquid or an inert gas.

Before stowing packages containing these cargoes an inspection should be made for signs of leakage or previous leakage.

While stowing, such cargoes should be kept away from inflammable solids, from cargoes which are dangerous when wet or corrosive. Such cargoes should be effectively segregated5 to avoid interaction with spontaneously combustible cargoes in the event of an accident6 but may be carried in the same hold or compartment or on deck if a minimum horizontal separation-of 3 meters projected vertically is provided. The packages of explosives, inflammable gases, inflammable liquids, oxidizing substances7 and radioactive substances should not be loaded in the same hold or compartment containing spontaneously combustible* substances.

The weight of each package should not exceed the maximum allowed weight9 100 kg.

First Aid. 1. Call a doctor immediately when someone may have inhaled the vapor, or has been splashed by the liquid. Notify the doctor that the vapor has been inhaled or has got on the skin.

2. When the liquid has got into the eyes immediately wash them with water.

3. If the skin has been contaminated with a poisonous liquid immediately wash with water and soap.

4. Clothing contaminated with a poisonous liquid must be taken off immediately.