Different types of cargo

Read through the following passage on the different types of cargo. Use the techniques that you have practiced so far to help you.

Merchant ships are designed to carry cargo. This cargo may be divided into two basic types: bulk cargo and general cargo. Bulk cargo consists of a single commodity. This commodity is usually carried loose. General cargo consists of a variety of goods. These goods are packed separately. Bulk cargo is carried in specially designed vessels, therefore stowage presents few problems. With general cargo stowage presents many problems, because each item has its own type of packaging and characteristics.

Bulk cargo can be divided into liquid or dry bulk cargo. Liquid bulk cargo is carried in tankers. Most tankers are designed to carry crude oil or its refined products, such as fuel oils. The oil is carried in tanks. These are connected by a system of pipes to a central manifold. The cargo is pumped on board at the loading port by shore pumps. At the discharging port the ship pumps the oil ashore using her own pumps. Dry bulk cargo is carried in bulk carriers. The cargo is carried in self-trimming holds. Dry bulk cargo includes grain, iron-ore, coal and sugar. It is loaded automatically by buckets on a conveyor belt system or through large tubes. Although the cargo stows itself, it is important to maintain the ship's stability and to make sure that the cargo will not move during the voyage. Dry bulk cargo is unloaded by huge grabs on cranes or by giant suction tubes.

General cargo can be divided into containerized, non-containerized and refrigerated cargo. Non-containerized cargo presents most stowage problems, because each commodity has its own type of packaging and characteristics. Goods may be in bags, bales, cases or steel drums. Individual pieces of machinery may not be packaged at all. Some cargoes such as tobacco and rubber have a strong odour and will taint delicate cargoes such as tea and rice. Other cargoes such as cement and fertilizers are dusty and leave a residue behind them. Heavy cargoes must not be stowed on top of fragile cargoes. This can cause problems if the heavy cargo has to come out first. General cargo is loaded by cranes and the ship's own derricks. Non-containerized cargo is carried in multi-deck vessels. To help with the problem of stowage many types of general cargo are now being put into containers of standard dimensions. A container is 8 feet high and 8 feet wide (2.44m x 2.44m) and is usually 20 feet or 40 feet (6.1m or 12.2m) in length. They are carried in specially designed container ships and loaded and unloaded by special cranes from the quayside. The containers are stowed both above and below deck. Perishable cargoes such as meat, fruit and dairy produce are carried in ships with refrigerated holds. These holds are designed to keep food at the correct temperature. Some food such as fish is frozen solid, other food such as fruit is only chilled. Mutton and lamb are stowed fore and aft, beef when chilled is hung on hooks. Eggs and butter are easily tainted. Fruit needs good ventilation. Refrigerated cargo is loaded by cranes and derricks.

Read through the passage below, which is a development of the reading passage in Reading Comprehension (A). Here defining and non-defining relative clauses have been used to rewrite some sentences and to add extra information to others. As you read through the passage, write in the margin Df or N-Df opposite each clause as appropriate. The first two have been done for you. Also circle the words and phrases that the words underlined refer to, to show the links between and within sentences.

Merchant ships are designed to carry cargo. The cargo which they carry may be divided into two basic types: bulk cargo and general cargo. The former consists of a single cargo, which is usually carried loose. The latter consists of a variety of goods, which are packed separately. Bulk cargo is carried in specially designed vessels, therefore stowage presents few problems. With general cargo stowage presents many problems, because each item, which is different in shape and size, has its own type of packaging and characteristics.

Cargo which is carried in bulk can be divided into liquid or dry bulk cargo. Liquid bulk cargo is carried in tankers. Most are designed to carry crude oil, which is transported to the refineries, or its refined products, such as fuel oils. The oil is carried in tanks which are connected by a system of pipes to a central manifold. The cargo is pumped on board at the loading port by shore pumps. At the discharging port the ship pumps the oil ashore using her own pumps, which may be of the reciprocating or the centrifugal type. Dry bulk cargo is carried in bulk carriers. The cargo, which includes grain, iron-ore, coal and sugar, is carried in self-trimming holds. It is unloaded automatically by buckets on a conveyor belt system or through large tubes. Although the cargo stows itself, it is important to maintain the ship's stability and to make sure that the cargo will not move during the voyage. Dry bulk cargo is unloaded by huge grabs on cranes or by giant suction tubes which are called elevators.

General cargo can be divided into containerized, non-containerized and refrigerated cargo. Cargo which is not in containers presents the greatest stowage problem, because each commodity has its own type of packaging and characteristics. Goods may be in bags, bales, cases or steel drums. Individual pieces of machinery which are large and awkward may not be packaged at all. Cargoes which have a strong odor, such as tobacco and rubber, will taint delicate cargoes, such as tea and rice. Cargoes which are dusty, such as cement and fertilizers, leave a residue behind them. Heavy cargoes must not be stowed on top of fragile ones. This can cause problems if the heavy cargo has to come out first. General cargo which is not in containers is carried in multi-deck vessels. To help with the problem of stowage many types of general cargo are now being put into containers of standard dimensions. Containers, which are 8 feet high and 8 feet wide (2.44m x 2.44m) and 20 feet or 40 feet (6.1m or 12.2m) in length, are carried in specially designed container ships and loaded and unloaded by special cranes from the quayside. The containers are stowed both above and below deck. Cargo which is perishable, such as meat, fruit and dairy produce, is carried in ships which have refrigerated holds. These are designed to keep food at the correct temperature. Some food, such as fish, is frozen solid, other food, such as fruit, is only chilled. Mutton and lamb are stowed fore and aft, beef when chilled is hung from hooks, which slide along rails. Eggs and butter, which are packed in cases, are easily tainted. Fruit needs good ventilation. Refrigerated cargo is loaded in the same way as general cargo which is non-containerized.