Roll-on/Roll-off Vessels

Ro-Ro vessels (double- or multi-deckers) are designed to carry all sorts of rolled vehicles and motor cars — by driving them on/off the ship — and unitized cargo and lengthy/bulky cargo units. The types of access and distribution facilites are numerous, each designed to serve a specific purpose, such as: stern/bow doors and side ports, the doors being normally hydraulically operated; access ramps, internal ramps; hoistable platforms', lifts; bulkhead doors. The actual link between the shore and the ship consists of a bridge ramp hinged at the shore end and supported at the other end in such a way that the ramp can be

adjusted to varying tide levels. The ramp should be sufficiently wide to provide at least two traffic lanes2 for the vehicles. The connection between the ramp and the ship is usually by a secondary ramp. The latter may be a door or an internal ramp. Loading may be effected through the stern/bow doors or side ports. On some medium-sized ships both bow and stern doors are provided, the ramp arrangement at the bow being similar to that at the stern. Such arrangement enables the vehicles to drive straight off the ship without having to be turned or reversed3; hence the expression4 — «drive-through» type vessel5.

Inside the vessel the vehicles are distributed throughout the cargo spaces by either internal ramps or hoistable platform. Where space is available, fixed or movable ramps are used, since they permit faster rates of loading than mechanically operated vertical lifts. The fixed ramp has the disadvantage of reducing valuable parking space on the deck above and below it. It is of greater advantage to use a hinged ramp which can be used as a closure for the deck above. On some RO-RO vessels lifts of varying sizes are designed to transfer loads from deck to deck.

If the stern bridge ramp is placed right in the direction of ship's fore-and-aft centre line, the ship can moor only stern to the berth. When the ramp is placed at an angle to the centre line, the ship can moor either stern to or alongside the berth (but only with one of the ship's sides). Such development in ship-to-shore access as slewing ramp enables the ship to load and discharge rolled cargo directly astern, over the port quarter or over the starboard quarter. The advantage of the slewing ramp is that it makes the ship completely independent of port facilities; the ship can berth in remote ports, estuaries, rivers and other locations where only one berthing position is possible. The slewing ramp moves round the stern through approximately 80° and is operated by winches. Storage of the ramp is directly astern, and the stern opening forward of the ramp is closed by a separate watertight door.

The advantages of RO-RO vessels: 1. independence of port facilities; 2. quick door-to-door service; 3. faster cargo handling rate and, consequently, reduced turn-round time; 4. as goods are generally in containers, the risk of damage or pilferage is reduced.

The disadvantage of RO-RO ships: 1. a considerable amount of cargo volume is occupied by vehicles and trailers; 2. a considerable space below the lower deck cannot be used for trailers.