DAMAGE TO CARGO

Considerable damage to cargo may result if due care is not paid to its handling. Lack of care1 at port of loading may result in damage. Sometimes the effect of damage is not immediately apparent2, it may begin to show3 during the voyage.

The following are the examples of damage owing to bad handling4:

a) Inefficient and improper slinging.

Case goods should be arranged more or less in like sizes before slinging. Heavy cases should not be slung with light cases. Canvas slings should be used for bagged goods rather than for rope slings.

For very small packages wire net slings are preferable.

With some classes of goods, as for example, bags of cement, cases of fruit, butter, edible goods, etc., the use of trays is the best method of handling and slinging. In all cases of slinging due attention should be given to the weight of the cargo in the sling. Excessive loads cause undue stress which apart from

damage to lifting gear, may also result in crushing packages at the bottom and sides of the sling.

b) The use of cargo hooks.

This practice is a frequent source of damage. The use of hooks is generally regarded as almost essential for heavy bulky cases6. The hooks mustn't be used with any bales of cotton, bagged cargo and drums of liquids.

c) Crushing.

Damage to cargo from crushing is mainly due to lack of care in stowing. Incorrect use of wooden dunnage, unsuitable space allocation of cargo, insufficient attention paid to the type of cargo being loaded and to the order of stowage, are potential factors causing damage from this source. Heavy bulky packages stowed over and with fragile packages will produce undue stress upon the latter and, with the motion of the ship, may cause them to collapse6. Fragile packages are considerably less liable to damage from crushing if they are given tweendeck stowage, where more efficient compactness of stow is possible.

Wooden dunnage, badly laid, is often the cause of crushing damage.

Light packages should be stowed on top of each other.

d) Cargo gear7.

Much damage to cargo results from slings contacting with hatch-coamings bulwarks and obstructions within a hold. This is due, in no small measure8, to careless winch work9. Acceleration and retardation of winch speed cause breakage of packages.

The following are some of the more important precautions which must be observed with all types of derrick rigging10 and cargo gear:

1. All gear including derricks and attachments must be regularly lubricated.

2. All wires and ropes must be in good condition. They should be free from frays" or brakes, not deformed and not unduly worn.

3. All shackles, hooks, blocks and pins should be of the same safe working load as the derrick they serve.

4. In no circumstances should a derrick be overloaded.

5. Careful winch handling is very important. Inconsistent rates of acceleration when heaving up12 and lowering cause great stress upon all the gear.