# MULTI-PURPOSE VESSELS

Nowadays universal vessels are gradually being transformed into multi-purpose vessels. Multi-purpose vessels, retaining versatility of universal vessels, that is their capability to carry different bulk and general cargoes, are becoming, to a certain extent, specialized. Thus, all multi-purpose vessels are adapted to carry standard containers. Some of them are also adapted to carry rolled vehicles or heavy/bulky cargoes. Multi-Purpose Ships for the Arctic

Operations in the Arctic require high ice-breaking capability. The «Norilsk» and her sister-ships are capable of moving continuously through ice over one meter thick with two-meter snow on it. As they are intended to sail independently of icebreaker assistance, they are designed to cope with all conditions prevailing in the Arctic.

The «Norilsk»-class ships are two-compartment full ice-breaking vessels. Their hull is mainly of steel suitable for temperatures down to — 50°C. They are designed to carry general cargo, lengthy items, pallets and unit loads of up to 80 tons, 20 ft containers (including refrigerated ones) and 40 ft containers, trailers, trucks, explosives and inflammable liquids on voyages to the northern outposts', and bulk coal, ore and grain in the midship holds or concentrates in special 10 ft containers carried two layers on the tank top and one layer on the tweendeck as well as sawn packaged timber on the upper deck on return voyages.

The cargo handling facilities comprise: deck cranes specifically designed for very cold conditions, wide hatches, a quarter ramp leading to the after tweendeck and allowing vehicles to move through bulkhead doors over a tweendeck and a 40 ton cargo capacity non-propelling hover craft2 for discharging cargo on the ice, water or marshy land3.

Hatch covers. — The three mid ship holds (Nos. 2-4) are fitted with twinned hydraulically operated folding hatch covers4. No. 1 hatch on the raised forecastle has a two piece cover and No. 5 hatch aft of the superstructure is a single piece hinged cover5 operated by wire from its crane. All these covers extend right to the edge of coamings to prevent the build-up of ice6.

The tweendecks are flush from the after end of No. 5 and up to the forward end of No. 2 and access to the hold below is by a combination of end and side-folding wire-operated covers7. The long No. 3 tweendeck covers have portable hatch beams8.

To ensure the watertight integrity9, hydraulically-operated sliding bulkhead doors10 5 m in width and 4.3 m in height are fitted throughout the tweendecks.

No. 1 hold, specially designed to carry explosives, has plastic faced timber linings" fitted on battens and in the deckheads and coamings. The hatch cover rollers are of spark-proof material12.

The weather deck covers are manually battened and the seals are of a special rubber which is compressible at — 60°C13.

Ventilation. — All cargo spaces are fitted with forced ventilation system14. The rate of air exchange15 is 6 times per hour in holds 2, 3, 4; in RO-RO tweendeck and in number 1 hold it is 20 times per hour.

Ramp. — The quarter ramp giving direct access to No. 5 tweendeck is set at an angle of 65°, to the centre line. It is 18 m long with a driveway 5 m wide. The ramp is hydraulically operated and can handle loads up to 56 tons. It can be used on wharves varying from 1 to 4 m in height, giving a maximum slope of 13° at 8.5 m draught. Many of the Arctic outposts have no port facilities, and there the ramp can be lowered directly onto the ice provided the ice is at least 1 m thick.

Cranes. — Conventional cargo handling is effected by means of four electro-hydraulic deck cranes (three of 22 ton capacity at 22 m outstretch and a twin 40 ton crane for hatches 2-3 at 20 m outstretch). The cranes are specifically designed for Arctic application16 with the machinery contained within the closed and heated housing17, the exposed components" of the cranes being of low temperature steel.

Environmental protection. — To prevent pollution of the Arctic environment an oil-fired incinerator19 is installed and a sewage treatment plant20 is fitted in the engine room to meet IMO's21 and other international and national regulations.