UNIVERSAL VESSELS

Universal vessels can carry practically any types of cargoes, including refrigerated goods and liquids (in special tanks). However, most of them are designed to carry certain types of goods. Thus, liners are intended to carry general cargo, packaged cargo, containers, and also, in dependence of the route, special, liquid and refrigerated cargoes. Their deadweight2 varies between 2,000 and 20,000 tons and the speeds between 10 and 22 knots. Tramps are designed, as a rule, to carry bulk cargoes and timber, but can also carry general cargo if necessary. These ships must be of sufficient size to carry a profitable cargo and must be able to cope with bad weather conditions in any ocean; they must not be too large to enter the smaller ports of the world; speed is necessary but it increases running costs3, so that a compromise between fuel consumption4 and speed is desirable. The modern tramp travels at between 12 and 15 knots. Its deadweight varies between 2,000 and 10,000 tons.

The cargo spaces are holds, tweendecks and deep tanks. Holds and tweendecks are used to carry general and bulk cargoes. Deep-tanks may be of two types. The first type is designed to carry liquids only. The second type is designed to carry liquid and dry cargoes in turn5. Empty deep tanks are also used for liquid ballast. Liners may have special cargo spaces (to carry valuable or dangerous goods etc.) and refrigerated cargo spaces.

Conventional multi-deck vessels have holds divided horizontally by one or two tweendecks known as upper and lower tweendecks. The bottom part of any hold with tweendecks is the lower hold.

Below is described a typical universal standard design vessel:

The vessel is of 14,700 dwt (9,000 gross and 6,000 net tons). Her overall length is 450 feet, beam 69 feet, draught 31 feet. Her five holds are arranged so that numbers one to four holds are forward of the superstructure and only number five hold is abaft it. The holds are closed with electrically operated automatic sliding type hatch-covers6. Each hold is served by its own 5-ton derricks mounted in pairs. A single un-stayed mast is located forward, between numbers one and two holds; there is a pair of un-stayed cargo (- Samson) posts7, between numbers three and four holds and another pair abaft the funnel with derricks for working number five hold. The winches are driven by electricity.

The hull is of the single tweendeck type with a raised forecastle and bridge superstructure of medium height.

With a single diesel engine and screw she has a service speed of 14.5 knots, her fuel consumption being around 22 tons per day. She provides accommodation for a crew of thirty.

Since late 60s universal vessels have been superseded8 by specialized vessels. But world shipping crisis of mid-70s made shipbuilders and ship-owners design new types of universal and specialized combined vessels, as specialized vessels proved to be unprofitable9 due to their inflexibility10. Thus a new generation of universal vessels, better designed and equipped, was born. It was then that multi-purpose vessels, which are further development of universal vessels, appeared". Nowadays universal and multi-purpose vessels make up about a half of all ships being built.