BULK CARGOES

The following cargoes are classified as bulk ones: ores, coal, salts, green (raw) sugar, grain, fertilizers. Bulk cargoes are loaded into holds in bulk.

Transportation of bulk cargoes requires special knowledge to deal with them, or special ships, such as ore carriers.

Iron, manganese1 and copper ores are so heavy that they should be given bottom stowage2.

While loading or discharging ore, Dockers must be supplied with3 special protective appliances4 (planks5, trays, finders) to protect the ship's boards and hold compartments from possible damage.

It is prohibited to throw off* the ore into the holds without any protective means.

Before loading salt all iron parts in the holds must be whitewashed7. This is done to save the metal parts of the ship from corrosion. Stevedores should not permit loading the salt of different sorts into the same hold.

Bulk cargoes are usually loaded in bulk evenly* along the whole length of the ship with sufficient trimming9 from portside to starboard.

Holds should be supplied with thermometers in each compartment to measure the degree of cargo heating, because coal has the property of self-ignition10.

Before loading sugar the holds must be thoroughly dried and ventilated. Shifting boards should separate grain in bulk.

If the weather is unfavorable (rain, snow, etc.) loading as well as discharging must be stopped, otherwise the bulk cargo will be spoiled.

The weight of the bulk cargo may be defined by means of a ship's deadweight scale.