GENERAL CARGOES

The most industrial products and semi-manufactured goods' belong to general cargoes. General cargoes are the most valuable ones. They require the expenditure of much labour2 while loading and unloading. Storage and transportation of a lot of tare are very expensive too.

General cargoes comprise goods of the following types: bagged, baled, barreled, boxed, metals and machinery, heavyweights, goods in packages, containers and timber cargoes.

General cargoes are usually transported differently packed3.

Units of especially heavy cargo are frequently carried on deck. Locomotives, lorries, crates of heavy machinery such as transformers and extremely large length of heavy timber (logs) and small consignments of general cargoes, which may damage other cargo, are also given deck stowage4.

Cargo carried on deck is shipped at «shipper's risk»5, unless contracted otherwise6, but nevertheless responsibility falls upon the ship to counteract7 to any possibility of loss or damage.

Proper means of fastening the cargo must be provided by lashings* Protection from the sun and weather can be obtained by the use of tarpaulins.

Let's consider some types of general cargoes, which are carried by dry-cargo ships.

Cotton cargo. It is shipped in tightly pressed bales but it is accompanied by fire risk9. The preparation of a hold or compartment for this cargo is primarily directed towards lessening the risk of fire. Cotton is liable to10 produce heat and is also subject to" spontaneous combustion. Any space in which cotton is to be stowed should be perfectly dry, well aired and absolutely free from any oil stains.

If cotton cargo follows a clean general cargo efficient sweeping and airing will be sufficient, but if cotton follows a coal or the like cargo, it is necessary to wash the space of holds, due attention being given to drying.

Rice cargo calls for12 a very elaborate and efficient system of ventilation for two reasons. Firstly, rice evolves a certain amount of carbonic acid gas, and secondly its moisture content leads to the sweating of the hold. Condensation will therefore drop on the cargo from certain points of ironwork, unless necessary precautions are taken13. The rice cargo is liable to heat fairly quickly, and this fact associated with loss of moisture, explains the loss of weight in transit varying from one to three per cent. The floor of the hold should be covered with thick battens laid athwart ships, and boards laid fore and aft.

Sugar cargo. Apart from14 the general preparations of cleaning and drying the hold sugar is a cargo which demands absolute freedom from dampness and avoidance of any contact with iron or steel. Sugar is carried mainly in bags. It may be either «dry» or «green»15. Dry sugar is more highly refined16 than green sugar, the former being almost free from syrupy content while the latter17 is of a wet nature18. Dampness will result in considerable drainage19 in the form of syrup, which will damage the remainder of the cargo20, but it causes the bags to stick together21 in the form of solid blocks. So «green» sugar and «dry» sugar should not be stowed together. The bulkheads and sides of the hold space should be covered with separation cloths22. Bare ironwork should be avoided.

Пояснения к тексту

1. semi-manufactured goods — полуфабрикаты;

2. expenditure [iks'penditfa] of much labor— затрата значительного труда;

3. differently packed — раздельно упакованный;

4. to give deck stowage — укладывать на палубе;

5. at shipper's risk — на риск грузоотправителя;

6. unless contracted otherwise — если в договоре не указано иначе;

7. to counteract — противодействовать; зд. предусмотреть;

8. to be provided by lashings — должно быть обеспечено найтовыми;

9. fire risk — риск пожара;

10. to be liable to — подвергаться, быть доступным (чему-л.);

11. to be subject to — подвергаться (воздействию, влиянию);

12. to call for — требовать;

13. to take precautions (against) — принять меры предосторожности;

14. apart from — кроме;

15. green sugar — сахар-сырец; неочищенный сахар, dry sugar — очищенный сахар;

16. highly refined — тщательно рафинирован (очищен);

17. the former ... the latter — первый (из двух)... последний;

18. wet nature — сырой по своей природе;

19. dampness will result in considerable drainage — сырость вызовет значительную утечку;

20. the remainder of the cargo — остаток груза;

21. to stick together — склеиваться вместе;

22. separation cloth [kb6]— сепарационная ткань.