Container Ships

The use of containers has continued to increase in recent years. Containerization means the consolidation of goods of single or mixed commodity1 into a freight container which may be closed or open and which, if in conformance to the recommendations2 of the International Standards Organization (ISO), would be 10 ft, 20 ft, 30 ft or 40 ft in length by 8 ft wide and 9 ft or 9 ft 6 inch high in external cross-section. A container is designed to be handled mechanically throughout the transit3. Specially designed cellular container ships and efficient port facilities are essential in order to take full advantage of the system. To support container ship operation a great deal of capital has been invested in depots, road and roll vehicles, ships, containers and port terminals.

The usual practice requires that container handling be removed to an area some distance from the pier; and, since it must be then moved a distance for the shiploading, it might just as well be moved to a point inland so that door-to-door service can be realized.

Such a service offers the maximum possibilities for reduction in overall transport costs, since individual containers are only handled twice; once when loading and once when unloading the container. Damage, pilferage, packaging, insurance and stock inventories4 are all potentially capable of reduction.

Ships that carry containers can range from the cellular type full container ship9 to semi-container ship where some of the cargo spaces can be used to carry containers.

The cellular type full container ship is equipped to carry containers in the holds and on the hatch covers. Within the holds of the vessel there is a cellular structure of angle bars' forming

container guides into which the containers are stowed, one on top of another. The containers carried on deck are carefully secured to ensure that they will not shift. The only movement of the container within the ship is vertical one, and thus loading and discharging is carried out by vertical movement only. Heights of container stacks vary and may be six high in the holds and three high on deck. In general, cells are arranged fore and aft in groups, their number depending on the ship's breadth. The distinguishing features7 of container ships are very large hatch opening and elimination of tweendecks8. The fore and aft groups of cells are separated by heavy web frames9, or similar structure, to act as support to the cells and also give rigidity10 to the vessel. Each cell is for one container stack and extends vertically from the coaming level to the tank-top. The basic functions of the cell-guide system11 are: 1. to facilitate the lowering and positioning of containers; 2. to distribute the lateral loads from the containers (arising from the rolling motion of the ship) to the adjacent hull structure12.

Cellular type full container ships are serviced by shore-based or shipboard gantry cranes. Cellular ships, operating between loadcentres equipped with shore-based cranes, have the advantage of greater deadweight capacity as compared to self-sustaining ships fitted with shipboard mounted gantry cranes.

In many areas of the world on short interport voyage service feeder ships of similar configuration but of reduced capacity are in operation. By fully mechanizing the handling of cargo and using shore based travelling gantry cranes the typical feeder ship can be turned around in about five hours13, a practical loading-unloading cycle being two to three minutes per container.

There are also roll-on/roll-off container ships on which the containers are mounted on the wheels or trailers and are then rolled on the ship and stowed complete with their undercarriage14. Such ships are particularly advantageous for short voyages, but they can carry fewer containers as compared to cellular full container ships or feeder ships of equivalent size because of considerable space occupied by trailers and access ramps from deck to deck. This difference may be as much as 35% or even more.

Like all other specialized ships designed to carry unitized cargo container ships have a high service speed ranging from 16 to 26 knots, and even up to 30 knots, in dependence of the ship's deadweight.