Bulkers

Bulkers are single-decked ships, i.e. ships with no tweendecks in their holds but fitted out with various types of special cargo handling equipment. The cargo is often shot straight into the hold1 and discharged by grabs2, pneumatic suction plants3 and other bulk handling methods. The modern ocean-going bulk carrier dates from the mid-fifties. Subsequent development has led to an increase in ship size, though not to the same extent as in tankers. There are bulk carriers of over 150,000 dwt today, but the largest of those in normal use are between 60,000 and 70,000 dwt, known as Panamax size4, while «handy size»5 bulkers are those in the 25,000-30,000 dwt range.

Cement Carrier «Sergo Zakariadze». The vessel is designed to carry cement, while other bulk cargoes, such as grain, ores, coal, sugar and bauxite can also be carried. The «Sergo Zakariadze» is a single-screw motor vessel with a bulbous bow, transom stern and six-tier superstructure. The hull is all-welded, of combined framing6. The ship has seven holds with a total capacity of 27,015 cu.m. Cargo holds Nos. 2, 3, 5 and 6 designed for the carriage of cement have a double skin along the sides and sloping bulkheads throughout the length of the holds. The cement holds' inner bulkheads as well as those separating the holds are corrugated7. The holds are sealed with mechanically operated folding-type hatch covers. The closing design comprises two pairs of covers, each pair folding lengthwise8 by means of a hydraulic cylinder. The hatch covers have four fillers9 to couple them with air chambers10 for handling cement.

On a ballast passage the vessel can take ballast in double-bottom tanks under the cargo holds, in underdeck tanks located in the upper part of the cargo holds as well as in the forepeak and afterpeak tanks. Ballasting operations are effected with the help of two onboard ballast centrifugal electricity driven vertical self-priming pumps11 with a capacity of 400 cu.m/h, while stripping is effected by an ejector of 100 cu.m/h driven by fire pumps. The pumps and the valves are controlled from a special panel in the Central Control Post12. Remote control of the filling or emptying of tanks is also available. All the double-bottom tanks are fitted with indicators showing the lowest level

in the tanks while drainage sumps13 in the holds have devices signalling w^hen a sump is full.

The main particulars of the vessel are: length o.a. -184.4 m, length b.p. — 172.0 m, breadth — 22.8 m, depth amidships — 14.1 m, summer draught -10.1 m, deadweight — 23,940 t, main engine output — 11,200 h.p., ballast speed — 15.5 knots, sailing range — unlimited.